Lithium-ion Incident Response 
Standard Operating Procedure

Nicholas Odeh 8/2026

Contents

Purpose	3

Scope	3

Definitions	3

Introduction	4

Conditions for Potential Incidents	4

Overcharging	4

Degradation	4

Physical Damage	4

Thermal Runaway	5

Fire, Explosion, and Leak Management	5

Effects of Lithium-ion cells	5

Managing Fire Incidents	5

Managing Explosion Incidents	6

Managing Explosion Incidents	6

Managing Leak Incidents	6

Evacuation	7

Medical Management	7

Health Effects	7

First Aid	7

Incident Documentation	8

FAQ	8

Lithium-Ion Incident Report	9

# Purpose

Establish a clear and systematic approach for identifying and managing lithium-ion related incidents.

# Scope

Form factors: Lithium-ion and lithium metal cells/batteries; batteries alone, packed with equipment, or contained in equipment.

Statuses: New, Used, EOL, and DDR (Damaged, Defective, or Recalled). DDR includes batteries with potential for dangerous and rapid combustion, or short circuit.

Incident Management: Guidelines for safely managing lithium combustion events, including containment procedures, extinguishing methods, and post-event cleanup.

Post-Incident Management: Recovery protocols, asset handling, and reporting requirements following containment and mitigation.

# Definitions

Cell/Battery: Includes both lithium-ion and lithium metal chemistries.

Equipment: Device/machine powered by lithium cells/batteries.

Damaged/Defective/Recall (DDR): Batteries identified as defective or with potential for dangerous evolution of combustion, or short circuit.

EOL: End of Life

SOH: State of Health

LIR: Lithium-ion Incident Report

# Introduction

Lithium-ion cells contain large amounts of energy, when that energy is released in an uncontrolled manner due to improper use, storage, or physical damage, it may cause a lithium-ion incident. Lithium-ion incidents pose unique hazards, including combustion, explosion, and toxic gas release. It is paramount that those working with lithium-ion cells review this SOP to enable a rapid incident response.

# Conditions for Potential Incidents

The below list contains common conditions that lead to lithium-ion incidents; this list is not exhaustive. If a Lithium-ion cell exhibits any of these conditions, immediately classify it as DDR and follow DDR handling procedures.

### Overcharging

Most modern lithium systems have overcharge protection. However, these systems can fail, allowing for the overcharge of lithium-ion cells. This causes higher resistance, leading to thermal instability. Never reuse an overcharge lithium battery.

Signs: Hot to touch, bulging, emitting an unusual odor or chemical smell.

### Degradation

Lithium-ion cells degrade over time leading to poor SOH. A cell's SOH is calculated based on the cell's calendar age and number of cycles.

Signs: Bulging cells, reduced battery life.

Calendar age: Amount of time elapsed since manufacture date of cell.

Cycle: Cycle degradation happens when you use and recharge the cell.

### Physical Damage

Punctured, deformed, and or otherwise damaged cells can catch fire or explode, potentially causing an incident with secondary ignition sources.

Signs: Visual damage, leaking cells, emitting an unusual odor or chemical smell.

### Thermal Runaway

Rapid and uncontrolled rise in temperature that can result in fire or explosion.

Signs: Bulging, hissing, smoking, rapid rise in temperature.

# Fire, Explosion, and Leak Management

Even after extinguishing a lithium-ion fire, there is a risk of reignition, and medical emergencies can develop from even short-term exposure of a lithium-ion incident. Fire extinguishers often do not have enough thermal mass to extinguish Lithium-ion fires. Water is the recommended method for fire suppression.

### Effects of Lithium-ion cells

It's important to understand what lithium-ion cells can do during an incident. The points below summarize key considerations to remember during such events.

- Lithium-ion cells contain flammable liquid that may vent and or ignite.

- Lithium-ion cells can burn rapidly.

- Lithium-ion cells can ignite other batteries in proximity.

- Lithium-ion cells can produce irritating, corrosive and/or toxic gases, including hydrogen fluoride gas.

### Managing Fire Incidents

The following should ONLY be done if safe to do so:

- Contact emergency medical service.

  - Inform emergency services personnel on the situation, including the materials involved.

- Keep unauthorized personnel away.

- Spray water or foam onto the base of the lithium-ion fire.

- Identify any subsequent fires developing in proximity of the initial fire.

- Move undamaged containers away from the area around the fire

- Ventilate area and stay upwind.

### Managing Explosion Incidents

The following should ONLY be done if safe to do so:

- Contact emergency services.

  - Inform emergency services personnel on the situation, including the materials involved

- Keep unauthorized personnel away.

- Spray water or foam onto the base of the lithium-ion fire.

- Identify any subsequent fires developing in proximity of the initial explosion.

- Move undamaged containers away from the area around the fire

- Ventilate area and stay upwind.

### Managing Explosion Incidents

The following should ONLY be done if safe to do so:

- Contact emergency services.

  - Inform emergency services personnel on the situation, including the materials involved

- Keep unauthorized personnel away.

- Spray water or foam onto the base of the lithium-ion fire.

- Identify any subsequent fires developing in proximity of the initial explosion.

- Move undamaged containers away from the area around the fire

- Ventilate area and stay upwind.

### Managing Leak Incidents

The following should ONLY be done if safe to do so:

- Eliminate all possible ignition sources from the immediate area.

- Do not touch or walk through leaked material.

- Contact emergency medical services if needed.

  - Inform emergency services personnel on the situation, including the materials involved.

- Keep unauthorized personnel way.

- Absorb lithium-ion leaks with sand or non-combustible material.

- Remove and isolate contaminated clothing.

- Leaking cells and contaminated material should be placed in inert material containers.

# Evacuation

In the event that the incident is too big, or too dangerous, execute the following evacuation protocol:

- Immediately exit through the closest emergency exit.

- Immediately contact local emergency response teams and deploy emergency alarms.

- Congregate upwind and begin basic triage on those affected.

# Medical Management

Lithium-ion incidents pose health risks that can lead to bodily harm or death. Always consult with a medical professional for an authoritative decision regarding medical treatment, the health effects and treatment methods listed in this SOP are not exhaustive and should not be used to replace treatment guidance from professional medical personnel.

### Health Effects

- Lithium-ion cells can produce toxic fumes that can cause dizziness or asphyxiation.

- Lithium-ion cells can produce hydrogen fluoride gas, a powerful corrosive & contact poison.

- Contact with lithium-ion electrolytes may irritate skin, eyes and respiratory tract.

- Lithium-ion fires can produce 3rd burns.

### First Aid

Always contact a medical professional immediately after lithium-ion incident, regardless if you believe you do not need medical attention. When contacting medical personnel, ensure they are aware of the materials involved. Below are first aid actions that are recommended during and after a lithium-ion incident, based on type of injury:

Dizziness and or unconsciousness:

- Move victim to fresh upwind air, if it can be done safely.

- If victim is unconscious, check airway and pulse. Perform CPR if necessary.

- Contact emergency medical personnel.

Contact with lithium-ion electrolytes:

- Remove and isolate contaminated clothing

- Immediately flush skin or eyes with running water for at least 20 minutes.

- Contact emergency medical personnel.

Contact with lithium-ion gases, including hydrogen fluoride:

- Move victim to fresh upwind air, if it can be done safely.

- Immediately contact emergency medical personnel for medical attention.

# Incident Documentation

For incidents involving lithium-ion cells, document the type of battery and the extent of the incident in the lithium-ion incident report (LIR) sheet provided at the end of this SOP. Retain the document for internal reference.

# FAQ

Is water safe for Lithium-ion fires?

Yes, water works as a fire extinguishing medium since the lithium inside of these batteries are a lithium salt electrolyte, and not pure lithium metal. Pure lithium is reactive with water, while lithium salts are non-reactive.

Where should I store the LIR?

A printed copy should be retained by the ITAM Lead in a secure location. Additionally, all LIRs should be stored electronically in a dedicated LIR folder within the organization’s SharePoint site.

# Lithium-Ion Incident Report

Lithium-Ion Incident Report

Name: __________________________
Employee ID:___________________
Date of Incident:_________________
Location:________________________

Type of Incident: ☐ Combustion   |   ☐ Explosion   |   ☐ Leak   |   ☐ Other: _________________________

Emergency Services Contacted: ☐ Yes   |  ☐ No

Medical attention needed: ☐ Yes   |  ☐ No

Name(s) of personnel that required medical attention:______________________________
____________________________________________________________________________
____________________________________________________________________________

Reason(s) for medical attention:_________________________________________________
____________________________________________________________________________
____________________________________________________________________________

Description of Incident:_______________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________
___________________________________________________________________________________

FOR INTERNAL USE ONLY